PROJECT REPORT IoT-Based Vehicle Theft Detection System ───────────────────────────────────── Shivam Sharma Student ID: 225EC50006 Department of Electronics & Communication Engineering ESP32 · MPU6050 · GPS/GSM · BLE · IoT

# Abstract

This report presents the design and implementation of an IoT-based Vehicle Theft Detection System developed to address the growing problem of unauthorized vehicle access in urban environments. The system integrates an ESP32 microcontroller, MPU6050 motion sensor (accelerometer + gyroscope), GPS module, GSM module (SIM800L), and Bluetooth Low Energy (BLE) to deliver a compact, low-power, and reliable security solution.

The system uses BLE proximity detection to distinguish between authorized and unauthorized usage. When the owner's device is out of range, the system enters Parking Mode and monitors for abnormal vibrations. Upon detecting suspicious movement, it immediately dispatches an SMS alert with a Google Maps location link to the registered phone number. Experimental results confirm high detection accuracy, negligible false-alarm rates, and consistent GPS and GSM performance under real-world conditions.

# Project Information

| Author | Shivam Sharma |
| --- | --- |

| Student ID | 225EC50006 |
| --- | --- |

| Domain | IoT / Embedded Systems / Security |
| --- | --- |

| Platform | ESP32 (Arduino Framework) |
| --- | --- |

| Technologies | BLE, GPS, GSM, MPU6050, C++ (Arduino) |
| --- | --- |

| Status | Completed — Hardware Tested & Verified |
| --- | --- |

# 1. Introduction

Vehicles have emerged as an essential mode of transportation in modern society, offering affordability, convenience, and environmental benefits. With increasing urban congestion, rising fuel costs, and growing awareness of sustainable mobility, vehicles are now widely used by students, working professionals, delivery personnel, and daily commuters. However, this surge in vehicle usage has also contributed to a parallel rise in vehicle thefts across cities and residential areas.

With the advancement of the Internet of Things (IoT), new opportunities have emerged to develop smart, automated, and connected security systems. GSM provides long-range, low-cost wireless communication, allowing systems to send alerts over standard mobile networks. GPS offers real-time location tracking with high accuracy, enabling owners to identify stolen vehicles in all weather and environmental conditions. When combined with microcontroller-based platforms, these technologies allow the creation of compact, portable, and energy-efficient security solutions.

This project addresses the gap in lightweight, low-cost vehicle security systems by combining BLE-based owner detection, accelerometer-based motion sensing, and GSM-based instant alerting into a single embedded solution.

# 2. Objectives

- Detect unauthorized movement of a parked vehicle using an MPU6050 motion sensor.

- Use Bluetooth Low Energy (BLE) to identify owner proximity and switch between Owner Mode and Parking Mode automatically.

- Transmit real-time GPS coordinates to the owner's mobile phone via SMS upon theft detection.

- Minimize false alarms using multi-threshold accelerometer filtering.

- Design a compact, low-power, and affordable hardware prototype suitable for real-world use.

# 3. System Architecture

The system is built around the ESP32 microcontroller, which acts as the central processing unit managing all peripheral modules. The architecture follows a dual-mode design pattern — Owner Mode and Parking Mode — that significantly reduces false alarm rates while ensuring responsive theft detection.

## 3.1 Block Diagram

| ⚡ Power Supply | ───► | 🔧 ESP32 Main Controller | ◄─── | 📡 Sensors GPS · Accel · Gyro |
| --- | --- | --- | --- | --- |
|  |  | ▼ |  |  |
|  |  | 📶 GSM / GPRS / VoLTE Communication Module |  |  |

Figure 1: System Block Diagram — ESP32 at center, sensors feeding in, GSM/GPRS pushing alerts out.

## 3.2 Operating Modes

| Feature | Owner Mode | Parking Mode |
| --- | --- | --- |
| BLE Status | Owner device connected | Owner device disconnected |
| Motion Monitoring | Inactive | Active (MPU6050) |
| LED Indicator | OFF | ON when theft detected |
| SMS Alert | Suppressed | Triggered immediately |
| GPS Tracking | Standby | Real-time location active |

Table 1: Comparison of Owner Mode and Parking Mode

# 4. Key Components

| Component | Function | Key Specification |
| --- | --- | --- |
| ESP32 WROOM-30 | Central microcontroller — processes all sensor data, manages BLE, GPS, and GSM modules | Dual-core 240 MHz, Wi-Fi + BLE |
| MPU6050 | 3-axis accelerometer + 3-axis gyroscope for motion detection | ACCEL_DELTA_G = 0.03g threshold |
| NEO-M8N GPS | Real-time geolocation tracking; provides latitude/longitude | High accuracy, all-weather GPS |
| SIM800L GSM | Sends SMS alerts with Google Maps link to the owner's phone | Works without internet via 2G |
| BLE (Built-in) | Detects owner's smartphone MAC address for proximity authentication | Scanning interval: 2000 ms |
| LED Indicator | Visual alert when theft is detected in Parking Mode | Built-in pin 2 (ESP32) |

Table 2: Hardware Components and Specifications

# 5. System Workflow

The system operates as a continuous loop following these steps:

| 1 | System Initialization ESP32 boots and initializes all modules: BLE advertising, GPS (Serial2), GSM/SIM800L (Serial1), MPU6050 via I2C, and built-in LED. |
| --- | --- |

| 2 | BLE Proximity Check The system continuously scans for the owner's registered Bluetooth MAC address. If connected, the system enters Owner Mode — LED stays off, alerts suppressed. |
| --- | --- |

| 3 | Parking Mode Activation If the owner's device disconnects for more than 5 seconds (PARKING_MODE_DELAY), the system transitions to Parking Mode — high alert status. |
| --- | --- |

| 4 | Motion Detection In Parking Mode, the MPU6050 is polled continuously. The total acceleration vector is calculated and compared against the previous reading. A delta exceeding ACCEL_DELTA_G (0.03g) signals theft. |
| --- | --- |

| 5 | Theft Alert Triggered On detection: theftFlag = true, LED turns ON (HIGH), and the system prepares an alert message with the GPS coordinates. |
| --- | --- |

| 6 | GPS Location Fetch The system reads the latest NMEA data from the GPS module. If a valid fix is obtained, real coordinates are used. Otherwise, fallback coordinates are included in the message. |
| --- | --- |

| 7 | SMS Dispatch Via AT commands (AT+CMGF=1, AT+CMGS), the SIM800L sends an SMS containing 'VEHICLE THEFT DETECTED' and a clickable Google Maps link: https://www.google.com/maps?q=LAT,LNG |
| --- | --- |

| 8 | Continuous Loop The system locks until manually reset. BLE scan, motion checks, and GPS updates continue in the background to capture any repeated events. |
| --- | --- |

## 5.1 System Flowchart

START

↓

◆ BLE Connected?

YES → Owner Mode (Safe — no alerts)

NO ↓

Parking Mode Active

↓

◆ Movement Detected? (MPU6050)

NO → Continue monitoring (loop)

YES ↓

Theft Flag = TRUE ¦ LED ON

↓

Fetch GPS Coordinates

↓

Send SMS via SIM800L (Google Maps link)

↓

System Locked — Awaiting Reset

# 6. Software Implementation

The firmware is written in C++ using the Arduino framework on PlatformIO. Key libraries used:

- BLEDevice.h / BLEServer.h — BLE server setup and MAC address-based authentication

- TinyGPSPlus.h — Parsing NMEA GPS data from Serial2

- Adafruit_MPU6050.h / Adafruit_Sensor.h — I2C interface for accelerometer/gyroscope

- Wire.h — I2C communication for MPU6050

## 6.1 Key Configuration Parameters

| Parameter | Value | Purpose |
| --- | --- | --- |
| OWNER_MAC | 42:58:E0:DC:C8:C6 | Owner smartphone BLE address |
| ALARM_PHONE_NUMBER | 8767314166 | SMS alert destination |
| PARKING_MODE_DELAY | 5000 ms | Grace period before arming |
| RECONNECT_INTERVAL | 2000 ms | BLE re-scan frequency |
| ACCEL_THRESHOLD_G | 0.05 g | Minimum detection threshold |
| ACCEL_DELTA_G | 0.03 g | Change delta for theft trigger |
| LED_PIN | 2 | Built-in LED indicator pin |

Table 3: Software Configuration Parameters

# 7. Results & Discussion

The system was evaluated under controlled laboratory conditions as well as real-world environments to assess accuracy, responsiveness, power efficiency, and reliability.

## 7.1 Motion Detection Accuracy

The MPU6050 sensor demonstrated excellent performance in differentiating between normal environmental disturbances and genuine theft attempts. During outdoor tests involving minor bumps, wind-induced vibrations, and user adjustments, no false alarms were triggered due to the multi-threshold filtering algorithm.

When the vehicle was lifted, dragged, or subjected to sustained movement, accelerometer readings exceeded the predefined delta threshold, correctly triggering the theft classification. The sensing algorithm proved robust across all test scenarios.

## 7.2 Mode Switching Performance

The dual-mode architecture performed with 100% consistency across all trials. Owner Mode was correctly activated whenever the registered smartphone connected via BLE, and Parking Mode activated reliably after the 5-second grace period upon disconnection. No misclassification of owner presence or absence was observed.

## 7.3 GPS & SMS Performance

| Metric | Observed Result | Status |
| --- | --- | --- |
| Theft Detection Accuracy | High (no false negatives observed) | ✅ Pass |
| False Alarm Rate | Zero during normal usage | ✅ Pass |
| GPS Fix Time | ~30–60 sec (first fix) | ✅ Pass |
| SMS Delivery Time | < 5 seconds after trigger | ✅ Pass |
| BLE Mode Switch Latency | 5 sec (PARKING_MODE_DELAY) | ✅ Pass |
| Fallback Coordinates | Activated when GPS unavailable | ✅ Pass |

Table 4: System Performance Evaluation Results

# 8. Conclusion

The IoT-based Vehicle Theft Detection System successfully demonstrates that combining ESP32, MPU6050 motion sensing, GPS tracking, and GSM communication can produce a practical, affordable, and reliable anti-theft solution. The dual-mode BLE-based architecture effectively eliminates false alarms while ensuring immediate response to genuine unauthorized movement.

Experimental validation confirmed high detection accuracy, fast GPS acquisition, and consistent SMS delivery under varying network conditions. The compact hardware design and low power consumption make this system readily deployable for everyday use on bicycles, motorcycles, or any small vehicle.

## 8.1 Future Enhancements

- Mobile App Integration — Real-time push notifications with live map tracking.

- Geofencing — Automatic alerts when the vehicle crosses a predefined boundary.

- Cloud Dashboard — Historical theft data logging and analytics via Firebase or AWS IoT.

- Solar-Powered Battery — Extend standalone operation using photovoltaic charging.

- Fingerprint / RFID — Secondary biometric owner authentication layer.

# 9. Skills & Technologies Demonstrated

| Embedded C++ Arduino / PlatformIO | IoT Architecture ESP32 BLE + GSM + GPS | Sensor Fusion MPU6050 Accel + Gyro | Real-Time Systems Interrupt-driven loops |
| --- | --- | --- | --- |
| GSM AT Commands SIM800L SMS Protocol | GPS Parsing TinyGPSPlus / NMEA | BLE Authentication MAC-based Proximity | Hardware Design Circuit Schematic & Wiring |

Table 5: Technical Skills Demonstrated in this Project

# References

[1] Kolban, N. (2019). Kolban's Book on ESP32. Free PDF Edition.

[2] InvenSense Inc. (2013). MPU-6000 and MPU-6050 Product Specification Rev 3.4.

[3] u-blox AG. (2020). NEO-M8 Series — u-blox M8 GNSS Modules Data Sheet.

[4] SIMCOM. (2015). SIM800L Hardware Design V1.00.

[5] Miessler, D. (2021). BLE Security and Privacy. GitHub Security Research.

Shivam Sharma ¦ 225EC50006 ¦ ECE Department IoT-Based Vehicle Theft Detection System ¦ ESP32 + MPU6050 + GPS + GSM